Virtual Memory:

- Why Virtual Memory?

  - Programmer’s view Vs H/W view of memory

- Processor’s View Of Memory:

  - What processor sees is called physical memory – memory contained in actual memory modules.

  - Addresses used by processor have 1:1 mapping to bytes/words in physical memory

- Program’s view of Memory:

  - Regions:- OS, Code, Data, Heap (grows downwards) & Stack (grows upwards)

  - Some contiguous regions of program’s view of memory are actually used by the program. And there is enormous region in the middle between the heap & stack that the program will never access unless the heap/stack grows downwards/upwards resp.

  - The above is virtual memory. Every process/program has its own virtual memory.

  - How can virtual memory (address of program’s view of memory) be mapped to a corr physical memory address?

- Virtual Memory  Physical Memory Mapping Introduction:

  - When a program generates a virtual address (by either doing a load or a store operation), the processor needs to address some physical address.

  - How does the processor map what the program is trying to access(Virtual memory address) TO what really should be accessed(Physical memory address)

  - Mapping every virtual memory address to physical memory address is not a feasible option.

  - Instead, virtual memory (program memory) can be divided into equal size chunks called PAGES. Each page is typically 4KB.

  - Physical Memory is divided into slots that can hold pages & these slots are called FRAMES.

  - Correlation:- Pretty much the physical memory behaves like a cache for the virtual memory in that it has certain number of slots/frames where we can put pages.

    - A page is kind of a memory block & frame is kind of a cache line

  - The OS decides when the VA to PA mapping needs to be done while actual mapping mechanism is called page table.

  - Page table says for each page in the process where is the page mapped to in its physical memory.

Quiz:

Ans:- frame size <=> page size

1)Page frames = Phy mem/page size => 2GB/4KB = 2^19 page frames.

2)We need one entry in page table for each page in virtual memory:

=> number of pages = virtual mem/page size = 4GB/4KB = 2^20 pages => 1 M entries in page table.

- Virtual memory of all the applications can significantly exceed the size of physical memory, some of the virtual memory that the applications think they have is on the DISK & not in physical memory.

Virtual to Physical Address Translation:

1)Virtual Address will be generated by a program instruction/code, lets say load instruction.

2)Virtual Address is broken down into 2 parts:- least significant 12 bits represent the page offset (tells us where in the page we are-also assuming typical page size is 4KB) & Virtual page number (which tells us which page in the process we are talking about).

3)We take the virtual page number(VPN) from VA & use it to index into the page table. The entry corresponding to VPN in the page table tells us the corresponding physical frame number in physical memory.

4)The physical frame number is put together with page offset to generate a physical address which is then used access the physical memory.

a)Multi-level page tables/Hierarchical page tables (Where is savings)?

b)TLB